The purpose of this project was to design and implement a functional prototype of a smart thermostat that meets SysTec’s business requirements and prepares the architecture for future cloud‑connected development. This reflection summarizes how the prototype supports the required peripherals, how the system will connect to the cloud via Wi‑Fi, and how three hardware architectures, Raspberry Pi, Microchip, and Freescale, compare in their ability to support the project’s needs.

The thermostat prototype requires support for an AHT20 I2C temperature sensor, GPIO buttons, PWM‑driven LEDs, a 16×2 LCD, and UART communication. The Raspberry Pi supports all of these peripherals natively through its Linux device drivers and Python libraries, making integration straightforward during prototyping. Microchip’s ARM‑based microcontrollers (such as the SAMD21, SAMD51, and PIC32MZ families) also support I2C, GPIO, PWM, UART, and LCD interfaces, but they require low‑level configuration in C/C++ and often rely on MPLAB Harmony for driver setup (Microchip, 2020). Freescale/NXP microcontrollers, including the Kinetis and i.MX RT families, provide robust industrial‑grade peripheral support with advanced timers, DMA, and high‑performance serial interfaces. However, like Microchip devices, they require manual firmware development and hardware abstraction layer configuration. All three architectures can support the required peripherals, but the Raspberry Pi enables the fastest and simplest implementation.

Cloud connectivity is a core requirement for SysTec’s future growth. The Raspberry Pi includes built‑in dual‑band Wi‑Fi and a full Linux networking stack, enabling direct use of MQTT, HTTPS, and REST APIs with minimal development effort (Pi). Microchip’s ARM‑based microcontrollers do not include native Wi‑Fi and require external modules such as the WINC1500 or ESP‑based chips. Integrating these modules involves additional hardware, RTOS support, and custom networking firmware. Freescale/NXP microcontrollers also lack integrated Wi‑Fi and rely on external wireless modules paired with FreeRTOS and vendor‑provided Wi‑Fi driver stacks (NPX, 2026). While both Microchip and NXP architectures can achieve cloud connectivity, they require more engineering effort compared to Raspberry Pi’s built‑in wireless hardware and high‑level software environment.

The thermostat prototype uses Python, multiple hardware drivers, a state machine framework, and a multi‑threaded display manager, all of which require substantial RAM and storage. The Raspberry Pi provides 1–8 GB of RAM and expandable microSD storage, making it the only architecture capable of running the existing Python‑based implementation without modification. Microchip’s ARM‑based microcontrollers typically offer 32 KB to 512 KB of RAM and up to 2 MB of Flash, which is sufficient for embedded C firmware but far below the requirements of a Python runtime and multitasking environment (Microchip, 2020). Freescale/NXP microcontrollers offer 256 KB to 1 MB of RAM and up to 4 MB of Flash, making them more capable than many Microchip devices but still not suitable for Python or Linux‑class workloads. As a result, only the Raspberry Pi meets the memory and storage requirements of the current software architecture.

In conclusion, all three architects can be configured to support a thermostat design that meets SysTec’s requirements. However, only the Raspberry Pi meets all requirements without major redesigns. Therefore, the Raspberry Pi is the most practical choice for SysTec’s current architecture and long‑term cloud‑connected goals.

# References

Microchip. (2020, Feb 28). PIC32MZ W1 and WFI32E01 Family. Retrieved from Microchip: https://ww1.microchip.com/downloads/en/DeviceDoc/PIC32MZ1025W104_MCU_and_WFI32E01_Module_Datasheet_RevA.pdf

NPX. (2026). i.MX RT1060: Crossover MCU with Arm® Cortex®-M7. Retrieved from NPX: https://www.nxp.com/products/i.MX-RT1060

Pi, R. (n.d.). Raspberry Pi 4 Tech Specs. Retrieved from Raspberry Pi: https://www.raspberrypi.com/products/raspberry-pi-4-model-b/specifications/